# Lab Assignment-2
102303130 {ADITYA CHAUHAN}

Q.1>Molecular Dynamics – Force Calculation:

Exceution Table:-

->Performance Statistics:-(perf stat)

->In this experiment, a parallel molecular dynamics simulation for Lennard-Jones force calculation was implemented using OpenMP for a system of 1000 particles with a cutoff distance of 2.5. The program computes inter-particle forces efficiently, and the observed execution times decrease consistently as the number of threads increases, indicating correct parallelization without functional errors.

The performance results show near-linear speedup up to 8 threads, achieving a speedup of 4.99×. Beyond this point, performance gains begin to saturate, with a maximum speedup of 5.68× observed at 12 threads and an efficiency of 47.3%. This reduction in efficiency at higher thread counts is attributed to increased memory access contention, cache misses, and synchronization overhead rather than flaws in the algorithm.

Hardware performance statistics collected using perf stat further confirm this behavior, showing significant cache activity and a higher cache miss rate at the core level. These results indicate that the performance bottleneck at higher thread counts is primarily due to architectural limitations of the system rather than incorrect implementation.

->VTUNE STATS

| Metric | Observed Value | Interpretation |
| --- | --- | --- |
| CPI (Cycles Per Instruction) | 0.58 | Low CPI indicates efficient instruction execution and good pipeline utilization |
| Effective Physical Core Utilization | ~14–22% (≈1.1–1.75 out of 8 cores) | Many physical cores remain underutilized due to cache stalls and synchronization overhead |
| Effective Logical Core Utilization | ~23–27% (≈3 out of 12 logical cores) | Hyperthreading provides limited benefit for this workload |
| Cache Bound (Overall) | ~35–37% of clock cycles | Significant portion of execution time is spent waiting for cache data |
| L1 Cache Bound | ~10–15% | Frequent accesses to L1 cache dominate memory behavior |
| L2 Cache Bound | ~0–1% | L2 cache is not a performance bottleneck |
| L3 Cache Bound | ~4–5% | Minor contention and data sharing at the L3 cache level |
| DRAM Bound | ~0–0.4% | Very little time is spent waiting for main memory |
| Observed DRAM Bandwidth | ~3.4–5.6 GB/s | Memory bandwidth usage is far below the hardware peak |
| Peak Platform Bandwidth | 57 GB/s | Available system memory bandwidth is underutilized |
| Cache Misses (LLC Miss Count) | ≈ 0 | Majority of memory accesses are satisfied by cache |
| Average Memory Latency | ~8 cycles | Low latency confirms cache-dominated access pattern |
| Vectorization | 0% | Code is scalar due to control flow, atomics, and data dependencies |

VTune memory analysis shows that the application is not DRAM-bound, as the observed memory bandwidth (~3.4 GB/s) is far below the platform peak of 57 GB/s. Instead, the program is mainly cache-bound, with a noticeable amount of time spent on L1 cache accesses. The very low DRAM-bound percentage confirms that main memory is not the performance bottleneck.

Microarchitecture analysis reports a low CPI (~0.58), indicating efficient instruction execution. However, physical core utilization remains low (14–22%), suggesting that performance is limited by cache stalls, synchronization overhead, and lack of vectorization rather than raw computation. Overall, OpenMP parallelization improves execution time, but scalability is constrained by cache behaviour and memory access patterns rather than compute capability.

Q.2>BIOINFORMATICS_DNA SEQUENCE ALIGNMENT:-

EXECUTION TABLE:-

WAVEFRONT PARALLELIZATION (ANTI-DIAGONAL)

ROW-WISE PARALLELISATION(SIMPLER , BUT LIMITED)_

PERFORMANCE STATISTICS:- (perf stat)

Wavefront (anti-diagonal) parallelization shows poor performance, with speedup dropping below 1 as thread count increases due to strong data dependencies and frequent synchronization. Row-wise parallelization performs slightly better at low thread counts but still exhibits limited scalability because of row dependencies and synchronization overhead. Performance statistics indicate moderate instruction throughput and noticeable cache activity, confirming that memory access patterns and algorithmic dependencies dominate execution and restrict parallel scaling in DNA sequence alignment.

VTUNE STATS-

| Metric | Observed Value (from your data) | Interpretation |
| --- | --- | --- |
| CPI (Cycles Per Instruction) | ~0.89 (Atom), ~1.58 (Core) | Moderate CPI; instruction execution is reasonable but affected by dependencies and control flow |
| Effective Physical Core Utilization | ~32% (Atom), ~68% (Core cycles split) | Core activity exists, but effective parallel work is limited due to synchronization and dependencies |
| Effective Logical Core Utilization | Limited benefit observed | Hyperthreading does not significantly improve performance for this workload |
| Cache References | ~6.25M (Atom), ~8.0M (Core) | Frequent cache accesses dominate execution |
| Cache Miss Rate | ~3.7% (Atom), ~5.7% (Core) | Moderate cache misses due to DP matrix access patterns |
| Memory Bound | Low | Execution is not stalled on DRAM accesses |
| Cache Bound (Overall) | Moderate | Performance is limited by cache-level accesses rather than computation |
| L1 Cache Behavior | Dominant | Most data accesses are served from L1 cache |
| L2 / L3 Cache Impact | Minor to moderate | Some contention exists, but not the primary bottleneck |
| DRAM Bound | ~0% | Application is not limited by main memory bandwidth |
| Observed DRAM Bandwidth | Low | Well below platform peak, confirming cache-dominated behavior |
| Average Memory Latency | Low (cache-level) | Confirms memory accesses are mostly satisfied by cache |
| Vectorization | 0% | No vectorization due to branches and strong data dependencies |
| GFLOPS | ~0 | Algorithm is memory- and control-flow bound, not compute-bound |

VTune analysis shows that the DNA sequence alignment application is not DRAM-bound, with most memory accesses being served from cache. The observed CPI values indicate moderate instruction efficiency, but effective parallelism remains limited due to strong data dependencies and synchronization overhead. Cache accesses dominate execution, with moderate cache miss rates caused by dynamic programming matrix traversal.

Although OpenMP parallelization is applied, wavefront and row-wise approaches exhibit limited scalability due to inherent algorithmic dependencies. The absence of vectorization and low GFLOPS further confirm that performance is constrained by memory access patterns and control flow rather than computational throughput. Overall, the results demonstrate that DNA sequence alignment is poorly suited for fine-grained parallel scaling on multicore CPUs.

Q.3 Scientific Computing - Heat Diffusion Simulation:-

-> STATIC SCHEDULING

->DYNAMIC SCHEDULING

->GUIDED SCHEDULING

->CACHE BLOCK VERSION(BLOCK VERSION:32)

Execution & Performance Statistics (perf stat)

The 2D heat diffusion simulation demonstrates good parallel scalability due to its regular grid structure and predictable memory access pattern. Each thread updates independent grid points, avoiding race conditions and enabling efficient OpenMP parallelization. Among the scheduling strategies, guided scheduling achieves the best performance, reaching a speedup of approximately 4.6× at 8 threads, while static scheduling benefits from low overhead for uniform workloads. The cache-blocked version further improves performance by enhancing cache locality and data reuse.

| Metric | Observed Value | Interpretation |
| --- | --- | --- |
| Speedup (8 threads) | ~4.6× | Good parallel scalability for a stencil-based workload |
| Efficiency (8 threads) | ~55–58% | Moderate efficiency loss due to cache and bandwidth limits |
| CPI (Cycles Per Instruction) | ~0.24–0.31 (derived from IPC ~3.3–4.2) | Efficient instruction execution and good pipeline utilization |
| Effective Physical Core Utilization | ~18–23% | Core utilization limited by memory access and synchronization |
| Cache Miss Rate | ~1.6–3.1% | Low cache miss rate indicates good spatial locality |
| Cache Bound (Overall) | Moderate | Performance limited mainly by cache accesses |
| DRAM Bound | < 1% | Application is not limited by main memory |
| Observed DRAM Bandwidth | Low–Moderate | Well below platform peak, memory bandwidth not saturated |
| LLC Miss Count | Very low | Excellent cache locality due to stencil computation |
| Average Memory Latency | Low (cache-level) | Most accesses are served from cache |
| Vectorization | Low | Limited SIMD utilization due to loop structure |
| GFLOPS | Moderate | Computation present but not the primary bottleneck |

VTune analysis shows that the heat diffusion application is not DRAM-bound, with memory stalls remaining low and most data accesses being served from cache. The cache-blocked version improves temporal locality by reusing neighboring grid values within the cache, which explains the reduction in execution time compared to the non-blocked versions.

The observed instruction throughput indicates efficient instruction execution, with a high proportion of cycles spent in useful computation. Low cache miss rates further confirm good spatial and temporal locality inherent to the stencil computation. Although vectorization is limited, thread-level parallelism and effective scheduling compensate for the lack of SIMD utilization.

Overall, Q.3 demonstrates a well-parallelized stencil-based workload, where performance gains primarily arise from OpenMP parallelism, cache-friendly access patterns, and appropriate scheduling strategies rather than raw memory bandwidth or aggressive vectorization. This makes the heat diffusion problem well suited for shared-memory multicore systems